Question for the EDA:

- Is there Artificial light or is it natural?

While artificial lighting clearly plays a major role, indicated by sharp light increases when people arrive, there's evidence of weak natural light during unoccupied days (e.g., Sunday). This suggests that natural daylight penetrates the space in small amounts, likely due to winter sun angles and possible side openings or translucent panels. Therefore, most of the light is artificial, but traces of natural light are detectable.

- Is there heating in the room?
Temperature data shows that room temperature remains relatively stable and rises with occupancy, particularly in the early morning. This suggests a baseline heating system active throughout the day, common in Europe. The sharper temperature increases in the morning likely reflect either:

- Additional heating triggered by occupancy (e.g., programmable HVAC), or

- Body heat accumulation from people.

Thus, heating is likely always on, but its effect becomes more noticeable as the room fills.

- Do CO₂ levels reflect changes in occupancy?
It doubled on a workday compared to a rest day. Other data points support similar conclusions, such as temperature and light.

- Can we determine when the break occurs by the time?
 Yes, the daily dip around 13:00 is consistent across temperature, CO₂, and humidity data, indicating a common break or lunchtime. The simultaneous drop across multiple variables reinforces this interpretation.

- If the light increases, does that mean other metrics like humidity or CO₂ will increase later?									        An increase in light may be followed by an increase in CO₂ or humidity, not because of the light itself, but because it often coincides with people entering the space.

- Humidity vs. Humidity Ratio: Why are the graphs different? And what do they mean?  The graphs differ because Humidity depends on temperature (relative measure), while Humidity Ratio reflects the actual water content in the air (absolute measure), making it more accurate for analyzing human occupancy effects. While higher humidity or humidity ratio may correlate with human presence (due to exhaled vapor and sweat), these variables alone are not sufficient to confirm occupancy.

🔄 Big Picture: Human Presence Drives the System

| Variable | Human-Related Effect |
| --- | --- |
| Occupancy | Triggers or correlates with everything below |
| Light | Often increases with occupancy (people turn on lights) |
| CO₂ | People exhale CO₂ → levels rise with more people |
| Humidity | People exhale moisture + body heat → increases relative humidity |
| Humidity Ratio | Reflects actual moisture in air → influenced by occupancy and ventilation |

🔍 1. Occupancy as the Trigger

When people enter a space:

- They breathe out CO₂ and water vapor.

- Their body heat contributes to thermal gain.

- They often use lighting and devices.

- Their presence affects air quality, temperature, and humidity.

💨 2. CO₂ Levels

- Primary indicator of human presence.

- Rises steadily with more people and poor ventilation.

- Used in HVAC systems to adjust fresh air intake dynamically.

Example: A CO₂ spike in a closed meeting room is a classic sign of multiple people with insufficient airflow.

💡 3. Light Levels

- Often rise when people are present (lights turned on).

- Can also be influenced by:

  - Natural sunlight (time of day)

  - Light automation (smart sensors)

- Light patterns can indicate occupancy schedules (e.g., people arrive at 8:00, leave at 17:00).

💧 4. Humidity (Relative Humidity %)

- Relative humidity increases as people:

  - Breathe (exhale water vapor)

  - Sweat (especially in warm rooms)

- Warm air holds more moisture → so high occupancy + high temperature = higher RH.

- Comfort range is typically 30–60%.

💧 5. Humidity Ratio (Absolute Humidity)

- Expressed in kg of water vapor per kg of dry air.

- Unlike relative humidity, this is not affected by temperature.

- It's useful for:

  - HVAC design and dehumidification control

  - Understanding actual moisture load from occupancy

Higher occupancy = higher humidity ratio unless ventilation removes moisture.

What was the average of each variable during the time of the experiment?

February 2015 (Europe - average)

Average outdoor temperature: Approximately 1.2°C.

Average indoor room temperature: Approximately 20–23°C.

Average relative humidity (RH): Around 65–75%, varying by region and weather conditions.

Average indoor CO₂ levels: Typically ranged between 600 ppm to 1200 ppm during occupied periods in enclosed indoor environments.